Local and State Permitting and Buffer Variance Requirements for Dams

### The intent of this document is to clarify the permitting and buffer variance requirements for the construction and maintenance of dams, including “emergency” scenarios.

In general, land-disturbing activities associated with the construction and maintenance of dams are subject to the permitting and buffer variance requirements delineated in the Georgia Erosion and Sedimentation Act (GESA), the NPDES General Permits for Storm Water Discharges Associated with Construction Activity and the Rules for Erosion and Sedimentation Control.  In addition, authorization may be required from the United States Army Corps of Engineers if the land-disturbing activities impact Waters of the United States, including jurisdictional wetlands and streams.

In all instances, regardless of exemptions, adequate erosion and sedimentation control measures (i.e., Best Management Practices) must be provided to protect the State’s soils and waters.

Scenario 1 – Construction and/or maintenance of dams for recreational and/or stormwater management ponds where the disturbed area of the project (including any disturbed areas flooded by the impoundment) is equal to or greater than one (1) acre:

- In an area with a certified Local Issuing Authority (LIA), a local Land Disturbing Activity (LDA) permit is required.

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a Notice of Intent (NOI), NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan (Plan).

- In an area with no certified LIA, a fee of $80 per acre disturbed(2) must be paid to the Environmental Protection Division (EPD).   In an area with a certified LIA, a fee of $40 per acre disturbed must be paid to EPD and a fee of $40 per acre disturbed must be paid to the LIA.  The NPDES General Permit fees are in addition to any LDA permit fees required by the LIA.

- In an area with a certified LIA, multiple copies of the Plan (as specified by the LIA) must be submitted to the LIA for review and approval or disapproval.  In an area with no certified LIA regulating the project, a single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- For projects with land disturbances equal to or greater than 50 acres, regardless of the existence of a certified LIA, a single copy of the Plan must be submitted to the appropriate EPD District Office.

- The maintenance of any manmade storm-water detention basin, golf course pond, or impoundment that is located entirely within the property of a single individual, partnership, or corporation that impacts the buffer of coastal marshlands as defined in O.C.G.A.12-7-3(2.1) is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-6(b)(17)(A)(vii).

- Buffer variance applications must be submitted to the EPD Watershed Protection Branch for review and approval or disapproval.  Incomplete applications will be returned to the applicant.

Scenario 2 – Construction and/or maintenance of dams for recreational and/or stormwater management ponds where the disturbed area of the project (including any disturbed areas flooded by the impoundment) is less than one (1) acre and located within 200 feet of any perennial State Waters(4):

- In an area with a certified LIA, a local LDA permit is required (O.C.G.A. § 12-7-17(8)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is not required if the project is not located within a larger common plan of development.

- In an area with a certified LIA, multiple copies of the Plan (as specified by the LIA) must be submitted to the LIA for review and approval or disapproval.

- The maintenance of any manmade storm-water detention basin, golf course pond, or impoundment that is located entirely within the property of a single individual, partnership, or corporation that impacts the buffer of coastal marshlands as defined in O.C.G.A.12-7-3(2.1) is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-6(b)(17)(A)(vii).

- Buffer variance applications must be submitted to the EPD Watershed Protection Branch for review and approval or disapproval.  Incomplete applications will be returned to the applicant.

Scenario 3 – Construction and/or maintenance of dams for recreational and/or stormwater management ponds “technically supervised” by the Natural Resources Conservation Service (NRCS) where the disturbed area of the project (including any disturbed areas flooded by the impoundment) is equal to or greater than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(7)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a NOI, NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan.

- Maintenance (excluding dredging), repair and/or upgrade of a dam is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(7) and Part IV(i)(8) of the NPDES General Permits).

- A fee of $80 per acre disturbed(2) must be paid to EPD.

- A single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- Buffer variance applications must be submitted to the EPD Watershed Protection Branch for review and approval or disapproval.  Incomplete applications will be returned to the applicant.

Scenario 4 – Construction and/or maintenance of dams for recreational and/or stormwater management ponds “technically supervised” by the Natural Resources Conservation Service (NRCS) where the disturbed area of the project (including any disturbed areas flooded by the impoundment) is less than one acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(7)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is not required if the project is not located within a larger common plan of development.

- Project is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(7)).

Scenario 5 – Construction of dams for public drinking water reservoirs where the disturbed area of the project (including any disturbed areas flooded by the impoundment) is equal to or greater than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(11)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a NOI, NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan.

- A fee of $80 per acre disturbed(2) must be paid to EPD.

- A single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- Project is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(11)).

Scenario 6 – Maintenance of public drinking water reservoir dams and shorelines where the disturbed area of the project is equal to or greater than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(11)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a NOI, NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan (Plan).

- A fee of $80 per acre disturbed must be paid to EPD.

- A single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- Project is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(11)).

- Projects such as boat launches and docks (including access ways) are not considered maintenance projects.  Buffer variance applications for these projects must be submitted to the EPD Watershed Protection Branch.

Scenario 7 – Maintenance of public drinking water reservoir dams and shorelines where the disturbed area of the project is less than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(11)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is not required.

- Project is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(11)).

- Projects such as boat launches and docks (including access ways) are not considered maintenance projects.  Buffer variance applications for these projects must be submitted to the EPD Watershed Protection Branch.

Scenario 8  - Maintenance of a non-erodible drainage structure on a dam, as defined in GESA (O.C.G.A. § 12-7-3(7)), and technically supervised by NRCS where the disturbed area of the project is equal to or greater than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(7)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a NOI, NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan.

- A fee of $80 per acre disturbed must be paid to EPD.

- A single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- Project is exempt from the State-mandated buffer requirements for non-trout waters (O.C.G.A. § 12-7-6(15)(A)(iv)).

Scenario 9 - Maintenance of a non-erodible drainage structure on a dam, as defined in GESA (O.C.G.A. § 12-7-3(7)), and technically supervised by NRCS where the disturbed area of the project is less than one (1) acre:

- A local LDA permit is not required (O.C.G.A. § 12-7-17(7)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is not required.

- Project is exempt from the State-mandated buffer requirements (O.C.G.A. § 12-7-17(7)).

Scenario 10 - Maintenance of a non-erodible drainage structure on a dam, as defined in GESA (O.C.G.A. § 12-7-3(7)), where the disturbed area of the project is equal to or greater than one (1) acre:

- In an area with a certified LIA, a local LDA permit is required.

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity(1) is required, which includes the submittal of a NOI, NPDES General Permit – Fee Form, and an Erosion, Sedimentation and Pollution Control Plan.

- In an area with no certified LIA, a fee of $80 per acre disturbed must be paid to EPD.   In an area with a certified LIA, a fee of $40 per acre disturbed must be paid to EPD and a fee of $40 per acre disturbed must be paid to the LIA.  The NPDES General Permit fees are in addition to any LDA permit fees required by the LIA.

- In an area with a certified LIA, multiple copies of the Plan (as specified by the LIA) must be submitted to the LIA for review and approval or disapproval.  In an area with no certified LIA regulating the project, a single copy of the Plan must be submitted to the EPD Watershed Protection Branch(3) for review and a second copy of the Plan must be submitted to the appropriate EPD District Office prior to or concurrent with the NOI submittal.

- For projects with land disturbances equal to or greater than 50 acres, regardless of the existence of a certified LIA, a single copy of the Plan must be submitted to the appropriate EPD District Office.

- Project is exempt from the State-mandated buffer requirements for non-trout waters (O.C.G.A. § 12-7-6(b)(15)(A)(iv)).

Scenario 11 – Maintenance of a non-erodible drainage structure on a dam, as defined in GESA (O.C.G.A. § 12-7-3(7)), where the disturbed area of the project is less than one (1) acre and located within 200 feet of any perennial State Waters(4):

- In an area with a certified LIA, a local LDA permit is required (O.C.G.A. § 12-7-17(8)).

- Coverage under the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is not required.

- In an area with a certified LIA, multiple copies of the Plan (as specified by the LIA) must be submitted to the LIA for review and approval or disapproval.

- Project is exempt from the State-mandated buffer requirements for non-trout waters (O.C.G.A. § 12-7-6(b)(15)(A)(iv)).

Emergency Projects - Projects that are determined to be an “emergency” by the EPD Safe Dams Unit are not subject to the State-mandated buffer variance requirements.  This includes dam repairs to relieve an imminent dam breach or repairs necessary immediately after a breach.  If EPD determines that the repair is an emergency but not an imminent danger to downstream properties the applicant may elect to enter into a consent order with EPD that will allow the repairs to proceed.   If the applicant does not elect to enter into a consent order then a buffer variance will be required to complete the repairs.

If the disturbed area of an emergency project is equal to or greater than one (1) acre, permit coverage under Part IV(A)(6) of the NPDES General Permits for Storm Water Discharges Associated with Construction Activity is required.

In all instances, regardless of exemptions, adequate erosion and sedimentation control measures must be provided to protect the State’s soils and waters.  If additional information is required, please contact Michael Berry with the EPD Erosion and Sedimentation Unit at (404) 651-8554 or Tom Woosley with the EPD Safe Dams Unit at (404) 651-8488.

An information guide, Building or Renovating a Pond in Georgia – A Pond Guide for Citizens, is also available for landowners in Georgia who want to build a new pond or to renovate an existing pond.  There are numerous laws and agencies regulating pond construction in Georgia.  This document identifies the multiple agencies available that provide assistance in planning and designing ponds.

(1)  The NPDES General Permit forms are located on the EPD website, www.epd.georgia.gov.

(2)  The disturbed acreage for a reservoir project includes any areas disturbed during clearing and subsequently flooded by the impoundment.

(3)  The EPD Watershed Protection Branch will review these Plans for deficiencies using the applicable Erosion, Sedimentation and Pollution Control Plan Checklist (including the delineation of State Waters and State-mandated buffers).

(4)  A certified Local Issuing Authority may enact a local Erosion and Sedimentation Control Ordinance that exceeds the acreage provisions of the Georgia Erosion and Sedimentation Act.